How to upload code to raspberry pi zero – RP2040

There are many ways to upload code, commonly the raspberry pi pico extension from vscode works but it is very slow at creating a new project and is not very efficient.

Skipping the official extension and going straight to the Pico SDK + CMake + VS Code (as a plain editor) is a very reasonable choice — it's faster, more transparent, and gives full control.

Steps to upload code on mac: (for the very first time):

- Install the toolchain, the cross compiler for Cortex-M0+ and cmake/ninja to build:

`brew install cmake ninja`

brew install --cask gcc-arm-embedded

- Get the Pico SDK and keep it in the home directory:

mkdir -p ~/pico-zero

cd ~/pico-zero

git clone -b master https://github.com/raspberrypi/pico-sdk.git

cd pico-sdk

git submodule update --init

- set and environment variable pointing to it (add the export line).

nano ~/.zshrc

export PICO_SDK_PATH=~/pico-zero/pico-sdk

Steps to upload code from Mac: (regular method):

- Create the blank project directory:

mkdir blinkProject

cd blinkProject

- Always copy the import helper to the directory:

cp $PICO_SDK_PATH/external/pico_sdk_import.cmake .

- Open in VS Code and create the main code files.

code .

- Create the CMakeLists.txt and blink.c file. Careful with the project names on the CMakelists

- Build the code:

mkdir build		#have to do this once

cd build			#have to do this once

cmake -G Ninja ..		#run for every upload

ninja			#run for every update

- Flash the board

- Manual method:

  Hold the BOOTSEL button on the board, plug in USB, then release.

  It mounts as a drive (usually RPI-RP2).

  Drag and drop the blink.uf2 file onto it - the board auto-resets and runs your code.

- Auto method (you have to manually reset the board after uploading):

brew install picotool

picotool load blink.uf2 -f